Newbee Check list -RD350 /400 "Been Sitting"

- Fuel system clean- tank rusty? Phosphoric acid treatment.

Cross over tube clean -pipe cleaners

Petcock clean replace gasket @ fuel selector

New fuel lines and filters

- Carburetor clean

Clean all passageways compressed air, Pilot, choke, nozzle, air, fuel

Inspect needle and seats -if visible x5 eye loop wear mark replace

Remove air screw, clean with steel wool tip

Remove needle Jet  [Nozzle] clean all debris off shaft, clear holes, hone cavity.

Set float height as manual stated RD350 = 15mm +/- 1mm RD400 = 23mm +/- 1mm

Remove Jet Needle from slides -clean with steel wool -note type and clip position

Clean pilot jet -guitar wire to clean all holes -shoot with air

- Compression - test both cylinders [dry] must be a minimum of 100psi both sides

Both sides should be within 5% of each other.

- Timing- NOTE: Battery must have at least 11 VDC

Inspect point -pitted (R&R) replace or repair file down if in limits

Set gap to 0.012" - 0.016" [0.014']

Using a dial indicator [metric preferred] set timing to 2.0mmBTDC

If motor work and pipes, big carbs 1.8mmBTDC -Retarded

- Install new plugs as recommended R5-RD350 B8hs, RD400 B8es

gap to 0.022" - 0.026"

- Start that mother up!!!!

If all was done correctly 1-2 kick should bring it to life.

- If Right pipe [sitting on bike] smokes more than left crank seals will need to be replaced

For GOD sake this is a 35-39 year old bike rubber parts wear out due to many years of    exposure.

If seals need to be replaced Jetting will be a bitch -left side FAT jetting right side LEAN jetting. If  the labyrinth seal is weak both sides will smoke.

- Noise in the engine - Slappy pistons will make noise- Ride the Bitch.

When you are riding and it slows down and will not start. Wait for 10 minutes then start and go home you have just experienced a seizure. Medication for this is a new top end! YEY!!

Off you and your wallet go, new tires, chain and sprockets, brake rebuilds, rubber parts, new odds and ends. Your Wife has to love you a lot or the bike for this to work... Once you ride a well tuned RD you will find it is not the fastest bike out there. But is sure is fun, power band is an addicting thing on these bikes

Have fun

JT    RD's Only